Núm. Robot IP

Waffle1

Waffle2 ssh ubuntu@10.10.73.220

Waffle3 10.10.73.222

Waffle4 10.10.73.223

Waffle5

Waffle6 10.10.73.225

Waffle7 10.10.73.226

Waffle8 10.10.73.227

Waffle9

Waffle10 ssh ubuntu@10.10.73.229

Waffle11 10.10.73.230

Waffle12 10,10,73,231

## Terminal PC —

Suponiendo que has descargado el ZIP corregido en ~/Downloads/proyecto_ros1_uno.zip:

cd ~/catkin_ws/src

unzip ~/Downloads/proyecto_ros1_uno.zip

chmod +x ~/catkin_ws/src/proyecto/mobil/uno_orchestrator_ros/uno_orchestrator_ros/scripts/*.py

cd ~/catkin_ws

catkin_make

source devel/setup.bash

rospack profile

## Terminal PC — configurar .bashrc

IP_PC = ifconfig

Obrir .bashrc:

gedit ~/.bashrc

export ROS_MASTER_URI=http://IP_PC:11311
export ROS_HOSTNAME=IP_PC

export TURTLEBOT3_MODEL=waffle

 source ~/catkin_ws/devel/setup.bash

source ~/.bashrc

## Terminal PC — entrar al robot

ssh ubuntu@IP_ROBOT

Contraseña: turtlebot

ubuntu@ubuntu:~$

.bashrc del robot:

nano ~/.bashrc

export ROS_MASTER_URI=http://IP_PC:11311

export ROS_HOSTNAME=IP_ROBOT

source ~/.bashrc

## Terminal 1 — PC — roscore

source ~/.bashrc

roscore

## Terminal 2 — PC entrando al robot — bringup del TurtleBot

ssh ubuntu@IP_ROBOT

Contraseña: turtlebot

source ~/.bashrc

roslaunch turtlebot3_bringup turtlebot3_robot.launch

## Terminal 3 — al robot — cámara

ssh ubuntu@IP_ROBOT

Contraseña:

turtlebot

Luego:

source ~/.bashrc

roslaunch turtlebot3_bringup turtlebot3_rpicamera.launch

## Terminal 4

export TURTLEBOT3_MODEL=waffle

roslaunch turtlebot3_bringup turtlebot3_remote.launch

## Terminal 5 — PC — navegación corregida con TF fix

NO dentro del robot:

source ~/.bashrc

cd ~/catkin_ws

source devel/setup.bash

export TURTLEBOT3_MODEL=waffle

roslaunch uno_wall_navigation uno_navigation.launch

## Terminal 6 — PC — detector YOLO

source ~/.bashrc

cd ~/catkin_ws

source devel/setup.bash

(Instalar ultralytics)

sudo apt update

sudo apt install pip

pip install ultralytics

roslaunch uno_detector_ros uno_detector.launch image_topic:=/camera/image/compressed

Comprobar: rostopic echo /uno_detector/detections

## Terminal 7— PC — RViz

En RViz:

Fixed Frame = map

Map

RobotModel

LaserScan con topic /scan

PoseArray o ParticleCloud si quieres ver AMCL

2D Pose Estimate

Y marca en el mapa per posar robot

Comprobacions

rostopic echo /amcl_pose -n1

rosrun tf tf_echo map base_footprint

## Terminal 8— PC — orquestador UNO

source ~/.bashrc

cd ~/catkin_ws

source devel/setup.bash

roslaunch uno_orchestrator_ros uno_orchestrator.launch